TOPS Technologies

CCNA — Network Troubleshooting

Assessment A1 — Complete Answers

Theme: Small Startup Office Network

Total Time: 3 Hours

Assessment 1

Section A — Scenario-Based Concept Application

1. Deep Packet Inspection (DPI)

- DPI works at Network and Transport layers of OSI model.

- It inspects packet contents deeply.

- Helps identify which application is using high bandwidth.

- Detects traffic bottlenecks and abnormal traffic.

- Helps startup improve internet performance.

- Example: Streaming or downloads consuming bandwidth.

2. DHCP and DNS Roles

DHCP

- Automatically assigns IP addresses to devices.

- Provides:

- IP Address

- Subnet Mask

- Default Gateway

- DNS Server

DNS

- Converts domain names into IP addresses.

- Example:

- google.com → IP address

Scenario Explanation

- Devices get IP successfully → DHCP working.

- Websites do not open → DNS problem.

3. Inter-VLAN Routing and Static Routes

- VLANs separate departments logically.

- Devices in different VLANs cannot communicate directly.

- Inter-VLAN routing allows communication between VLANs.

- Router or Layer 3 switch performs routing.

- Static routes manually define traffic paths.

- Helps departments communicate securely.

4. ICMP vs HTTP/HTTPS Performance

ICMP

- Used by ping command.

- Sends small packets.

- Tests basic connectivity.

HTTP/HTTPS

- Used for websites and web traffic.

- Requires:

- DNS lookup

- TCP handshake

- Server response

Explanation

- Ping can be fast while website is slow.

- Website latency may occur due to:

  - DNS delay

  - Server overload

  - Web traffic congestion

5. Firewall ACL and Placement

- ACL stands for Access Control List.

- ACL filters network traffic.

- Can permit or deny specific traffic.

- Firewall protects network from unauthorized access.

- Firewall should be placed:

  - Between departments

  - Between LAN and Internet

- Example:

  - Block staff VLAN from accessing management VLAN.

Section B: Practical Application

Task 1: VLSM Subnet Scheme — 3 Departments, 15 Hosts Each (192.168.10.0/24)

Calculation Logic

- 15 hosts per department requires at least 17 total addresses (15 hosts + 1 network address + 1 broadcast). A /28 provides only 14 usable addresses — insufficient. A /27 provides 32 total addresses and 30 usable hosts, which comfortably accommodates 15 devices with room for growth.

  - Subnet mask for /27 = 255.255.255.224

  - Binary: 11111111.11111111.11111111.11100000

  - Block size = 32 addresses per subnet

### Subnet Table

| Department | Network Address | Subnet Mask | Usable Range | Broadcast | Prefix |
| --- | --- | --- | --- | --- | --- |
| Dept 1 (Sales) | 192.168.10.0 | 255.255.255.224 | .1 – .30 | 192.168.10.31 | /27 |
| Dept 2 (Finance) | 192.168.10.32 | 255.255.255.224 | .33 – .62 | 192.168.10.63 | /27 |
| Dept 3 (HR) | 192.168.10.64 | 255.255.255.224 | .65 – .94 | 192.168.10.95 | /27 |
| Remaining space | 192.168.10.96 | — | .96 – .254 | 192.168.10.255 | Future use |

## Task 2: Configure VLANs 10, 20, and 30 on a Cisco Switch

The following CLI commands create each VLAN, assign access ports, and configure an uplink trunk port for inter-VLAN routing:

Switch> enable

Switch# configure terminal

! Create VLANs

Switch(config)# vlan 10

Switch(config-vlan)# name Sales

Switch(config-vlan)# exit

Switch(config)# do wr

Switch(config)# vlan 20

Switch(config-vlan)# name Finance

Switch(config-vlan)# exit

Switch(config)# do wr

Switch(config)# vlan 30

Switch(config-vlan)# name Management

Switch(config-vlan)# exit

Switch(config)# do wr

! Assign physical interfaces to VLANs

Switch(config)# interface FastEthernet0/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

Switch(config-if)# exit

Switch(config)# do wr

Switch(config)# interface FastEthernet0/2

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 20

Switch(config-if)# exit

Switch(config)# do wr

Switch(config)# interface FastEthernet0/3

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 30

Switch(config-if)# exit

! Configure trunk port to router

Switch(config)# interface GigabitEthernet0/1

Switch(config-if)# switchport mode trunk

Switch(config-if)# switchport trunk allowed vlan 10,20,30

Switch(config-if)# exit

Switch(config)# do wr

! Verify

Switch# show vlan brief

Switch# show interfaces trunk

## Task 3: DHCP Pool for VLAN 10 Subnet (192.168.10.0/27)

Router> enable

Router# configure terminal

! Reserve gateway and management IPs from the pool

Router(config)# ip dhcp excluded-address 192.168.10.1 192.168.10.5

! Create the DHCP pool

Router(config)# ip dhcp pool VLAN10_POOL

Router(dhcp-config)# network 192.168.10.0 255.255.255.224

Router(dhcp-config)# default-router 192.168.10.1

Router(dhcp-config)# dns-server 8.8.8.8

Router(dhcp-config)# exit

Switch(config)# do wr

! Verify

Router# show ip dhcp pool

Router# show ip dhcp binding

On a Layer 3 switch, the VLAN 10 SVI acts as the gateway:

L3Switch(config)# interface vlan 10

L3Switch(config-if)# ip address 192.168.10.1 255.255.255.224

L3Switch(config-if)# no shutdown

Task 4: Wireshark — DNS Query and HTTP GET Header Analysis

Step 1 — Capture Traffic

- Open Wireshark, select the active network adapter, and click Start Capture. Browse to any HTTP website (e.g., http://example.com) to generate traffic.

Step 2 — Filter and Identify DNS

- Apply the display filter: dns

- In the DNS query packet, expand Domain Name System (query). You will find:

- Source IP = client PC IP (e.g., 192.168.10.5) — the device making the request

- Destination IP = DNS server (e.g., 8.8.8.8) — the resolver being queried

- Queries field = the domain name requested (e.g., example.com)

- In the DNS response, the Answers section shows the resolved IP address

Step 3 — Filter and Identify HTTP

- Apply the display filter: http

- Click on an HTTP GET packet and expand Hypertext Transfer Protocol. You will find:

- Source IP (in the IP header) = client's IP address

- Destination IP = web server's IP address

- GET / HTTP/1.1 — the request line confirming the method and path

- Host: field = domain name (e.g., example.com)

Section C — Integrated Case Study

1. Most Likely OSI Layers

- Layer 1 (Physical Layer)

- Layer 2 (Data Link Layer)

- Reason

- Packet loss

- Flapping connectivity

- Switching issues

- Cable faults

- Broadcast storms

2. Bottom-Up Troubleshooting Sequence

- Check physical cables and connections.

- Verify switch port status and LEDs.

- Check VLAN and interface configuration.

- Monitor traffic and spanning-tree status.

3. CLI Commands

- show interfaces

- show spanning-tree

- show mac address-table

4. Switching Loop

- Protocol: Spanning Tree Protocol

- STP prevents loops in switched networks.

- Failure reasons:

- STP disabled

- Wrong configuration

- Redundant links

- PortFast misuse

5. Long-Term Configuration Changes

- Implement proper VLAN segmentation.

- Enable STP on all switches.

- Use Port Security.

- Apply Storm Control.

- Separate departments into different broadcast domains.

- Monitor network regularly.